Adafruit NeoPixel Library – Quick Reference Sheet

For Arduino + RGB LED Strips using the Adafruit NeoPixel library.
Used to control individually addressable RGB LEDs (like WS2812 / NeoPixels).

strip.setPixelColor(pixelNumber, r, g, b) or strip.setPixelColor(pixelNumber, colorValue)

Parameters

- pixelNumber → LED index (0 to numberOfPixels-1)

- r → Red value (0–255)

- g → Green value (0–255)

- b → Blue value (0–255)

- colorValue → 32-bit color made using Color() or ColorHSV()

What it does: Sets the color of ONE pixel in memory. ⚠️ The LED will not update until you call show().

strip.show()

What it does: This is what pushes your instructions out to the LEDs. You must call this after setting pixels to actually see the LEDs change.

strip.Color(r, g, b)

Parameters

- r → amount of Red (0–255)

- g → amount of Green (0–255)

- b → amount of Blue (0–255)

Returns: 32-bit color value that you can pass into setPixelColor()

What it does: Combines RGB values into a special color value 
that can be stored in a variable or used in setPixelColor().

🧠 Important Reminders

- Pixel numbering starts at 0

- Changes don’t appear until show() is called

- RGB values range from 0–255   (int)

- HSV hue ranges from 0–65535   (unsigned int)

Example

uint32_t purple = strip.Color(255, 0, 255);

strip.setPixelColor(0, purple);

strip.ColorHSV(hue, saturation, value)  or  strip.ColorHSV(hue)

Parameters:

- hue → 0–65535 (color wheel position)

- saturation → 0–255 (color intensity, from grey to full color)

- value → 0–255 (brightness)

Returns: 32-bit color value that you can use with setPixelColor()

What it does: Creates a color using HSV (Hue, Saturation, Value) instead of RGB. Great for rainbow animations!

🔁 Basic Animation Pattern

Most animations follow this pattern:

- Set pixel colors

- Call show()

- Delay or repeat

for( int i = 0; i < num of pixels; i++){

strip.setPixelColor(i, 255, 0, 0);

strip.show();

delay(50);

}

strip.fill(colorValue) or strip.fill(colorValue, firstPixel, count)

Parameters:

- colorValue → 32-bit color

- firstPixel → Starting LED number

- count → Number of LEDs to fill

What it does: Sets multiple LEDs to the same color at once.

Example: This will fill the entire strip green-

strip.fill(strip.Color(0, 255, 0));

strip.show();

strip.clear()

What it does: Turns all pixels off (sets them to black in memory). Remember to call show() afterward.

strip.setBrightness(brightnessLevel)

Parameters: brightnessLevel →  integer number from 0–255

What it does: Sets overall strip brightness. ⚠️ Should usually be set in setup() before lighting LEDs.